#                                         A d v a n c e d

#                  ����  �����    ����  ��    �� �  �����

#                 �  ��  �    �  ��  �� ��    �� � ������

#                 �    � �����  ��    �� ��  ��  �       �

#                  ����  �   �  �      �  ����   �  �����

#                 Advanced Gravis Computer Technology Ltd.

#

#     Fax (604) 431-5155                   BBS  (604) 431-5927 V32bis

#     Compuserve: GO PCVENB (71333,350)    InterNET EMail: tech@gravis.com

#     FidoNET: 1:153/978                   SBCNET: 13:900/3

#     AOL: Adv Gravis                      Genie: Page M805 Topic 6

#                     MEGA-EM VERSION 2.03 - 01/19/94

#                               USERS MANUAL

#        Copyright (C) 1993 by Advanced Gravis Computer Technology Ltd.

#                           All Rights Reserved

#                 0.0 Quick Readme Information

#                 1.0 Legal Information

#                 2.0 Introduction

#                 3.0 Installation

#                 4.0 Mega-Em Usage

#                 5.0 Compatibility Problems With Various Software

#                 6.0 Solutions to Commonly Encountered Problems

#                 7.0 Answers to Commonly Asked Questions

#                 8.0 Mega-Em Revision History

#0.0.0 ************ CHAPTER 0. - QUICK README INFORMATION *****************

#All internet support questions regarding Mega-Em should be sent via email

#to: jayeson@gravis.com (Jayeson Lee-Steere)

Quick notes: You must run MEGAEM, then EMUSET before any emulation is

active.

Always run EMUSET between programs. This will reset Mega-Em

internally. Many games require the sound devices to be in a

power on state to work correctly.

Should you choose to use a real Sound Blaster instead of

Mega-Em's emulation, you must run MEGAEM with the /SBOFF

switch.

All known compatibility problems are listed in chapter 5

of this manual.

Remove any debuggers from memory before loading Mega-Em.

#             IMPORTANT: Adaptec SCSI users must read section 4.1.2 of

#             this manual before using Mega-Em.

#1.0.0 **************** CHAPTER 1. - LEGAL INFORMATION ********************

This software and documentation are protected by copyright law,

with all rights reserved. Copyright is held by:

Advanced Gravis Computer Technology Ltd.

#101, 3750 North Fraser Way

Burnaby, B.C. V5J 5E9

CANADA

This software is provided as-is. Advanced Gravis makes no warranties,

express or implied, and does not warrant that the software will be fit

for any particular purpose. Advanced Gravis will in no event be liable

for loss of profits, nor incidental or consequential damages. You use

this software at your own risk.

This software may not be modified, disassembled or reverse engineered

in any way without written authorization from Advanced Gravis.

Mega-Em may be distributed in unmodified form freely to all Gravis

UltraSound owners, provided no charge is made for it, with the exclusion

of BBS (Bulletin Board System) service charges.

All trademarks and registered trademarks are the property of their

respective owners.

#2.0.0 ****************** CHAPTER 2. - INTRODUCTION ***********************

Mega-Em is a software program which allows you to emulate sound devices

other than those present in your system. Mega-Em allows you to emulate

the Roland MT-32 and Sound Canvas and the Sound Blaster (Digital Only)

using a Gravis UltraSound (GUS) sound card. Sound Blaster Music and Roland

MPU-401 MIDI interface emulation through the UltraSound MIDI port is

planned for the future.

Mega-Em has the following features:

* High Quality Roland Sound Canvas and MT-32 music emulation.

* Sound Blaster Digital (DAC) emulation, with generally improved sound

quality over a real Sound Blaster card.

* Loads entirely into the EMS memory space - uses no low/upper DOS memory.

* Fast - entirely written in assembler, utilizing the 386 instruction set

* High level of customization, while still retaining ease of use.

Mega-Em currently requires the following to function correctly:

* A 386 or better AT class personal computer.

* A hard drive.

* A Gravis UltraSound card.

* An Expanded Memory Manager, such as EMM386.EXE, QEMM386 or 386MAX.

* Approximately 64k of free Expanded Memory (EMS). Mega-Em uses no low DOS

memory (i.e. Memory in the 1st megabyte).

#NOTE: Adaptec SCSI users MUST read section 4.1.2 of this manual.

#3.0.0 ****************** CHAPTER 3. - INSTALLATION ***********************

To install Mega-Em to your hard drive do the following.

Step 1. Make and enter a directory where you wish to install Mega-Em. For

example if you wish to install to the directory C:\MEGAEM, type

the following:

C: <ENTER>

MD \MEGAEM <ENTER>

CD \MEGAEM <ENTER>

Step 2. Unzip the Mega-Em distribution archive into this directory. For

example if the archive is on a floppy disk in drive A:, you would

type:

PKUNZIP A:\MEGEM* <ENTER>

Step 3. It is now recommended that you add the Mega-Em directory to your

path. Only experienced users should try other methods such as

creating aliases for the executables.

To add this directory to your path do the following:

Edit your AUTOEXEC.BAT file (in the root directory of your boot

drive) using a text editor. eg:

EDIT AUTOEXEC.BAT <ENTER>

Look for a line that starts with PATH= or SET PATH=. eg:

PATH=C:\DOS;C:\ULTRASND

Add the directory name to the end of this line. For the above

example the line should look like this:

PATH=C:\DOS;C:\ULTRASND;C:\MEGAEM

Save the file.

Reboot your computer.

Mega-Em is now installed and ready to use.

#4.0.0 ****************** CHAPTER 4. - MEGA-EM USAGE **********************

#  4.1.0  Loading Mega-Em and Configuring The Emulation

#    4.1.1  Overview

#    4.1.2  Mega-Em Command Line Options

#    4.1.3  Emuset Command Line Options

#    4.1.4  The Default Options Files

#  4.2.0  Configuring Software For Use With Mega-Em

#  4.3.0  A Step By Step Example

#4.1.0 LOADING MEGA-EM AND CONFIGURING THE EMULATION.

#4.1.1 OVERVIEW

Mega-Em consists of two main parts. The first being the MEGAEM.EXE

executable which must be loaded first to provide your system with

emulation capability. This program should be considered a driver: just

loading this alone will not allow you to emulate anything. The second is

the EMUSET.EXE executable which allows you to turn emulations on or

off, and control which devices Mega-Em will emulate (e.g. MT-32, Sound

Canvas and/or Sound Blaster).

For example, first load MEGAEM.EXE by typing:

MEGAEM [options] <ENTER>

Valid MEGAEM.EXE options are listed in section 4.1.2, and are optional.

Assuming there were no problems, you should see INSTALLATION SUCCESSFULL

printed on the screen, and the resident driver portion of Mega-Em will

now be in memory. In most cases you should be able to leave MEGAEM.EXE

loaded indefinitely, switching emulation on and off as needed with

EMUSET.EXE. You can uninstall Mega-Em from memory at any time by typing

MEGAEM -F.

Now when you wish to run a program requiring emulation, type:

EMUSET [options] <ENTER>

Valid EMUSET.EXE options are listed in section 4.1.3, and are optional.

Mega-Em will now be configured for the emulation you specified, and you

are ready to run your software.

#Note that when emulation is active, Mega-Em hides the UltraSound from any

#other programs. Be careful to disable emulation by typing EMUSET OFF

#before running any 'native' UltraSound software.

In addition it is recommended that you run EMUSET between running

different software programs, as this will reset Mega-Em internally.

Confused? Read on. There is a step by step example in section 4.3

#4.1.2 MEGAEM.EXE COMMAND LINE OPTIONS

The following options can be added to the MEGAEM.EXE command line. As you

can see all options should be preceded by a '-', but can also be preceded

by a '/', or just separated by a space.

-H or -?  Display the MEGAEM.EXE help screen, which is basically a

shortened version of this section of the manual.

-U or -F  Uninstall Mega-Em from memory. Can also be referred to as

removing or freeing Mega-Em from memory. This option should

normally only be used with no other options on the command line.

-ROFF     Turns off Roland emulation.

-SBOFF    Turns off Sound Blaster emulation.

-RIRQx    Specify Roland Irq number. By default Mega-Em will not generate

any Roland Irqs since very little software requires it. If your

software fails on Roland initialization, you should use this

option, preferably using Irq 2. Valid values are x=2,3,5 or 7.

-NOEMS    Forces Mega-Em to load when EMS services are disabled (for

example, when then NOEMS option is used in your EMM386 command

line). Note that you must still have a memory manager loaded to

run Mega-Em. Caution must be exercised when using this option,

since Mega-Em will use EMS to activate your memory manager. You

must manually make sure your memory manager is active to use

this option. Adding EMM386 ON to your AUTOEXEC.BAT will, for

example, achieve this. If you have programs loaded into upper

memory, the memory manager will most likely be active.

-RGDT     Relocate server GDT. This switch may be needed on some systems

with certain memory managers for Mega-Em to operate correctly.

You should only use this option if the system crashes as

MEGAEM.EXE loads, or the system becomes very unstable after

MEGAEM.EXE has been loaded, or MEGAEM.EXE suggests you use it.

It is possible that using this option may cause the above

mentioned problems. Use with caution.

-FGDT     Force server GDT expansion. This switch may allow Mega-Em to be

used with system/memory manager combinations which cause

MEGAEM.EXE to respond with 'EMM Incompatibility Error: Can not

expand server GDT'. There is however a good chance that using

this option will crash the system as MEGAEM.EXE loads, or

renders the system unstable after MEGAEM.EXE has loaded. This

option has precedence over the -RGDT option. Use with caution.

-P        Enable Adaptec (or compatible) SCSI controller protection.

#          IMPORTANT: Adaptec SCSI users must read this. Failure to read

#                     and understand this option may result in loss of data

#                     on SCSI drives. If you are not using an Adaptec SCSI

#                     device ignore this option.

Mega-Em requires special consideration when used with some

Adaptec SCSI devices. Many Adaptec SCSI controllers are factory

set to operate at an I/O base address of 330 (hex). Check your

controller manual, if your controller is not set to this address

you do not need to use this option. If you can change the base

address of your card, it is recommended that you do so and avoid

using this option.

What's the big deal you ask? Well unfortunately 330 (hex) is

also the default base address for the Roland MPU-401 MIDI

interface (i.e. Roland sound devices). This means any software

attempting to access a Roland card will instead access the SCSI

controller, resulting in the controller crashing, and possible

loss of data. In addition, without the -P option, Mega-Em will

consider any SCSI drivers to be Roland software, stopping the

SCSI controller from getting any commands, resulting a

guaranteed system crash.

Mega-Em overcomes the problem by distinguishing between Roland

and SCSI software. Any software loaded into the low 640k memory

area BEFORE Mega-Em, and any software in the ROM BIOS/High/Upper

Memory area is considered to be SCSI software. Any software

loaded into the low 640k memory area after Mega-Em is considered

to be Roland software.

This is not ideal. You must never load any SCSI software which

accesses the controller directly into low memory after Mega-Em

(including AFDISK.EXE). More importantly you must never load any

Roland software into High Memory, or before Mega-Em.

If you do not correctly understand what this option does and are

using an Adaptec SCSI device, please do not use Mega-Em until

you do understand.

The following options are not listed on the Mega-Em help screen. They are

primarily for debugging purposes, and are included here since they may be

of use to some users.

-SBBxxx   Specify Sound Blaster base address. By default Mega-Em will use

the UltraSound base address. This is primarily a debugging

feature allowing Mega-Em's Sound Blaster emulation to operate

over the top of a real Sound Blaster. Note that the occasional

DMA failure (and hence system crash) may occur with a real Sound

Blaster installed in the system. Valid values are xxx=210,220,

230,240,250 or 260.

-SBIx     Specify Sound Blaster Irq. Another debugging feature purely for

testing Sound Blaster software with different Irqs. By default

Mega-Em will use the UltraSound MIDI Irq. Valid values are

x=2,3,5 or 7. 7 is the recommended value, since a number of

programs are 'hard coded' to use this value.

-MEMx     Manually specifies the amount of memory on the UltraSound card.

There should be no need to use this option, since Mega-Em should

correctly auto-detect the amount of memory on the UltraSound

card.

x=1,2,3,4 for 256k,512k,768k,1024k respectively.

#4.1.3 EMUSET.EXE COMMAND LINE OPTIONS

The following options can be added to the EMUSET.EXE command line. As can

be seen all options should be preceded by a '-', but can also be preceded

by a '/', or just separated by a space.

-H or -?  Display the EMUSET.EXE help screen, which is basically a

shortened version of this section of the manual.

-OFF      Turns emulation off. The default is to turn emulation on. For

example just typing EMUSET <ENTER> will activate the emulation,

with default options. Typing EMUSET -OFF <ENTER> will then

disable any emulation.

-MT       Emulate a Roland MT-32/LAPC1. The default is to emulate a Roland

Sound Canvas. You should use this option if your software does

not have Sound Canvas/SCC1/General MIDI support, but does have

MT-32/LAPC1/Roland support. ("Roland" alone usually refers to

the MT-32/LAPC1 devices).

-L        Enable the UltraSound line input. This option can be useful in

some situations, such as when using a real Sound Blaster card.

-Mxx      Specify music volume level. The default level is 12. The valid

range is xxx=1-16. For the technically minded, this switch

adjusts the volume in 3dB increments.

-Vxx      Specify master volume level. The default level is 16. The valid

range is xxx=1-16. For the technically minded, this switch

also adjusts the volume in 3dB increments.

-MONO     Set music playback to mono. The default mode is stereo playback.

This option can be useful since a few pieces of music will cause

audible crackling during very fast stereo pans.

-SS       Causes sustained instruments to slowly fade away. With some

software programs, the notes will become stuck and without this

option, become very annoying.

-COn      Coexist with real MPU-401 MIDI interface. While Mega-Em will

function correctly with a real MPU-401 MIDI interface installed

in the system, no software will be able to access the real

interface while emulation is active and these options are not

used. These options allow Mega-Em to be used together with a

real MPU-401 MIDI interface. This can be useful for DOS based

MIDI sequencers. Note that when using this option, Mega-Em no

longer emulates the MPU-401 interface, however will intercept

all data sent to it.

-CO1  Allow both input and output to real MPU-401. With this

option music will play through both the external MIDI

device(s) and the selected Mega-Em output device.

-CO2  Only allows input from real MPU-401. Music will only play

through the selected Mega-Em output device. Using this

option may also increase Mega-Em's compatibility on

systems with a real MPU-401.

The following options are not listed on the EMUSET help screen. They are

generally not needed, but are provided since they may be of use to some

users.

-VOxx     Specify the number of voices used for emulation output. The

valid range for xx is 14 to 32, with the default being 22. Using

a larger number of voices increases the number of simultaneous

notes that can be played, but also lowers the output sampling

rate and hence the sound quality.

-Fxx      Sets the sampling rate at which Mega-Em stores samples in the

UltraSounds memory.

-F22 Strips the samples to half the normal sampling rate of the

UltraSound patches, which is approximately 22kHz. This

approximately doubles the number of patches which can be

loaded, however also results in noisier music playback.

This is the default for 256k UltraSound cards.

-F44 Loads the samples at the normal sampling rate of the

UltraSound patches, which is approximately 44kHz. This

results in higher quality music playback. This is the

default for 512k, 768k and 1024k UltraSound cards.

-Xx       Specify Sound Blaster version to emulate. The default is -X1,

emulate original Sound Blaster.

-X1 Emulate Original Sound Blaster. Some software (including

newer Sierra) works better if you use this option.

-X2 Emulate Sound Blaster 2.0. This option is provided only for

completeness. No software tested has required this option,

including a number of programs which state they do require a

Sound Blaster 2.0. Some newer Sierra games will not work

with this option.

#4.1.4 THE DEFAULT OPTIONS FILES.

Since many users will use some options all of the time (such as -P),

Mega-Em supports the ability to make these default. For both MEGAEM.EXE

and EMUSET.EXE you can create a single line text file containing options

you would like to be automatically added to the command line. The files

should be named MEGAEM.OPT and EMUSET.OPT respectively.

For example if you want always use the -P option, create a single line

text file called MEGAEM.OPT in your Mega-Em directory, where the first

line reads:

-P

These text files can be created with any text editor, including the EDIT

program supplied with DOS 5 or DOS 6.

#4.2.0 CONFIGURING YOUR SOFTWARE FOR USE WITH MEGA-EM.

Software programs are configured for different sound devices in many

different ways. Some will attempt to auto-detect sound devices and use

the best one it finds, others will prompt you during the running of the

program. Most common is the use of a setup program, usually called SETUP,

INSTALL or CONFIG. Others require you to delete a configuration file,

after which you will be able to change the configuration. For the correct

procedure for a specific program, you should consult the program's manual.

Ok, I know how to configure my software, now what?

Well first make sure Mega-Em is loaded and emulation is on. If the program

is of the type that auto-detects, just run it and it should detect the

devices you are emulating. If you are prompted to select sound devices,

select the ones you have set up your emulation for.

But there are too many options there. What do I choose?

For the Roland MT-32 emulation you should look for 'MT-32' or 'LAPC1'.

Some software just has 'Roland' as an option. This usually refers to the

MT-32. Remember that if you select this option you must run EMUSET -MT,

otherwise Mega-Em will by default emulate a Sound Canvas.

For the Roland Sound Canvas emulation you should look for 'Sound Canvas',

'SCC1' or 'General MIDI'.

If you have the option of MT-32 or Sound Canvas, in most cases selecting

Sound Canvas will give better performance.

For Sound Blaster emulation look for Sound Blaster (strangely enough).

Remember Mega-Em will not play Sound Blaster Music, only digital effects.

Now the program is asking for my sound card settings. What should they be?

If prompted for a MIDI interface, you should select Roland MPU-401.

If prompted for a Roland base address, you must select 330 (hex).

If prompted for a Roland IRQ setting, you should select IRQ 2/9. If your

software fails to initialize the Roland, try using the -IRQx switch with

MEGAEM.EXE.

If prompted for a Sound Blaster base address, select your UltraSound Base

address unless you have used the -SBBxxx switch with MEGAEM.EXE.

If prompted for a Sound Blaster Irq, select your UltraSound MIDI Irq

unless you have used the -SBIx switch with MEGAEM.EXE.

If prompted for a Sound Blaster DMA channel, you must select 1.

You should now be ready to run your software. Always remember to load

Mega-Em and turn emulation on before running the software. Many programs

will crash if set up for a music device, and are then unable to locate it.

#4.3.0 A STEP BY STEP EXAMPLE.

This example assumes you have already installed Mega-Em on your hard

drive.

In addition, it is assumed you have consulted you program's manual and it

states you configure your sound options with a program called INSTALL, and

the program is run by typing START <ENTER>.

Step 1.  Load MEGAEM.EXE by tying MEGAEM <ENTER>. If you need to use any

options type MEGAEM options <ENTER>.

Step 2.  Turn on emulation by typing EMUSET <ENTER>. Many programs will

only allow you to select sound devices it can detect.

Step 3.  Load you program's installation program by typing INSTALL

<ENTER>.

Step 4.  Answer any prompts until you are prompted to select a sound

device.

Assume you are given the following options:

AdLib

Sound Blaster

Roland MT-32/LAPC1

Roland Sound Canvas

Roland MT-32/LAPC1 with Sound Blaster

Roland Sound Canvas with Sound Blaster

Step 5.  In this case you should select 'Roland Sound Canvas with Sound

Blaster' as it will probably give the best music performance and

also give digital sound effects. Note that any of the above

examples will work with Mega-Em, although the 'AdLib' option will

not give any sound, and the 'Sound Blaster' option will not play

any music or 'FM' effects.

Step 6.  Complete the rest of the prompts from the installation program.

If prompted whether or not to save the configuration, select YES.

Step 7.  Load your program by typing START <ENTER>. That's it.

Note: You should not have to configure the program again. Just make sure

you have loaded Mega-Em and turned on the emulation by running EMUSET.

#5.0.0 **** CHAPTER 5. - COMPATIBILITY PROBLEMS WITH VARIOUS SOFTWARE *****

#  5.1.0  Software that requires 386 protected mode and can not run under a

#         memory manager.

#  5.2.0  Software which is incompatible with Mega-Em.

#    5.2.1  Incompatibilities with all of Mega-Em's Emulations.

#    5.2.2  Incompatibilities with Roland Emulation

#    5.2.3  Incompatibilities with Sound Blaster DAC Emulation.

#  5.3.0  Specific software with known compatibility problems.

#    5.3.1  Harpoon (Roland)

#    5.3.2  Darkseed (Sound Blaster DAC)

#    5.3.3  Kings Quest I, Firehawk, Stellar 7 (Sound Blaster DAC)

#    5.3.4  Microsoft Windows in Enhanced Mode.

#    5.3.5  SBOS and UltraMID.

#    5.4.6  Lemmings 2 (Roland).

#    5.3.7  The Incredible Machine (General MIDI).

#    5.3.8  Budokan (Roland).

#    5.3.9  Vectordemo (Sound Blaster DAC)

#    5.3.10 Older Sierra Titles (Sound Blaster DAC)

#    5.3.11 Netroom 3

#    5.3.12 NHL Hockey

#5.1 SOFTWARE THAT REQUIRES 386 PROTECTED MODE AND CAN NOT RUN UNDER A

#    MEMORY MANAGER.

A few high-end games use the 386 protected mode to achieve superior

performance. Since many of these require full control of the system they

will not operate if the system is already in protected mode, such as when

a memory manager is loaded (excluding HIMEM.SYS and several shareware

EMMs).

Mega-Em uses the power of 386 protected mode for its emulation, and hence

the aforementioned games are not compatible with Mega-Em. Examples of

known games which exhibit such problems are COMANCHE and ULTIMA 7.

The only current solution is to use SBOS to emulate a Sound Blaster, or if

the game supports the use of standard AIL (Miles) drivers, use the

UltraSound AIL drivers. Note that the AIL drivers used by ULTIMA 7 are not

standard.

Furthermore, most software which use protected mode under a memory manger

will also fail with Mega-Em. This is due to the fact that system control

is taken away from Mega-Em. Examples of such software are Privateer,

Strike Commander, Syndicate and Windows.

#5.2.0 SOFTWARE WHICH IS INCOMPATIBLE WITH MEGA-EM.

#5.2.1 INCOMPATIBILITIES WITH ALL OF MEGA-EM'S EMULATIONS.

Software Title          Reason                         Solution

Comanche                Protected mode conflict        Use SBOS

Doom                    Protected mode conflict        Use Native Support

Dracula Unleashed       Protected mode conflict        Use SBOS -x2

IndyCar Racing          Protected mode conflict        Use SBOS -o3

Links 386 (Pro)         Protected mode conflict        Use SBOS

Privateer               Protected mode conflict        Use SBOS

Rebel Assault           Protected mode conflict        Use Native Support

Shadowcaster            Protected mode conflict        Use SBOS

Strike Commander        Protected mode conflict        Use SBOS

Syndicate               Protected mode conflict        Use SBOS

Ultima 7                Protected mode conflict        Use SBOS

Windows                 Protected mode conflict        Use UltraSound

Windows drivers

#5.2.2 INCOMPATIBILITIES WITH ROLAND EMULATION.

Software Title          Reason                         Solution

Legend Text Adventures  Needs smart mode compatibility Use SBOS(No Roland)

Terminator 2029         Unsure                         Use AIL drivers

#5.2.3 INCOMPATIBILITIES WITH SOUND BLASTER DAC EMULATION.

Software Title          Reason                       Solution

DMP (using -C1 option)  Non-standard DMA operation   Use GUS Support

Flight Simulator 5      Possibly buggy sound driver  May work with SBOS

Mortal Kombat           Protected mode conflict      Use SBOS

Panic                   Non-standard DMA operation   Use SBOS

Second Reality          Non-standard DMA operation   Use GUS Support

#5.3.0 SPECIFIC SOFTWARE WITH KNOWN INCOMPATIBILITIES.

#5.3.1 HARPOON (Roland).

While Harpoon does appear to have an MT-32 sound option, it does not

appear to use the Roland MPU-401 MIDI interface. Almost all other

software which supports Roland sound devices uses the MPU-401 interface.

Hence this is what Mega-Em uses for its emulation, and unfortunately is

not compatible with Harpoon.

#5.3.2 DARKSEED (Sound Blaster DAC)

You may find Mega-Em gives unstable performance with Darkseed. The Sound

Blaster DAC drivers appear to be buggy. There is an update patch available

for Darkseed and this should correct any problems.

#5.3.3 KINGS QUEST I, FIREHAWK, STELLAR 7 (Sound Blaster DAC)

All of these games have Sound Blaster drivers which will fail to

initialize on fast machines. This problem occurs even with a real Sound

Blaster. To overcome the problem run EMUSET with the -FAST switch. In

addition the above games have drivers hard coded to base address 220,

Irq 7.

#5.3.4 MICROSOFT WINDOWS IN ENHANCED MODE.

When Microsoft Windows is run in enhanced mode it uses an undocumented

interface to communicate with Expanded Memory Managers. While Windows will

run correctly in enhanced mode with Mega-Em loaded, upon exiting to DOS

the system will crash.

To prevent this from happening, Mega-Em is designed to detect when Windows

tries to enter enhanced mode, offering the user the option of using

standard mode, or exiting to DOS. Note that Mega-Em's emulations will not

work with Windows.

In future revisions of Mega-Em, this problem may be corrected, provided

some form of documentation for the interface used can be sourced.

#5.3.5 SBOS AND ULTRAMID.

You must not load either Mega-Em, SBOS or UltraMID into memory at the same

time. Doing so results in the programs' trying to use the UltraSound at

the same time, resulting in impaired performance, and possibly a system

crash.

This should not prove to be a problem since all three programs can be

removed from memory to allow the loading of one of the others. To remove

Mega-Em from memory type MEGAEM -f. To remove SBOS from memory, type

SBOS -f. To remove UltraMID from memory, type ULTRAMID -f.

Mega-Em is designed to automatically remove UltraMID and SBOS from memory.

UltraMID is designed to automatically remove SBOS from memory.

#5.3.6 LEMMINGS 2 (Roland)

With Mega-Em loaded, Lemmings 2 may have palette (colour) problems such as

abnormally flashing objects. Currently the only solution is to use a

different memory manager, or try removing other resident programs.

#5.3.7 THE INCREDIBLE MACHINE (General MIDI).

You may find that Mega-Em produces no output when The Incredible Machine

is configured for General MIDI (i.e. Mega-Em is configured for Sound

Canvas emulation). This should not be a major problem however, since The

Incredible Machine works correctly if you select MT-32 as your sound

device.

#5.3.8 BUDOKAN (Roland).

To use Budokan successfully with Mega-Em, you should load it by typing

BUDO Mt32 <ENTER>. If you choose Roland sound from within the game, you

may get no sound output, and the system may be rendered unstable.

#5.3.9 VECTORDEMO (Sound Blaster DAC)

This program may not always detect the Sound Blaster Irq correctly. To

overcome the problem, you should always run the install section of the

demo before executing the demo.

#5.3.10 OLDER SIERRA TITLES (Sound Blaster DAC)

If you are having problems with driver initialisation on early Sierra

titles and have a fast computer, run EMUSET with the -FAST switch. This

should overcome the problem.

#5.3.11 NETROOM 3

Mega-Em is not compatible with Netroom's cloaking features. To disable

these features run CUSTOMIZ, select CUSTOM SETUP and turn of system and

video cloaking in the custom setup menu. Note that the cloaked utilities

(DPMI server, disk cache e.t.c.) supplied with Netroom can not be used

with Mega-Em.

#5.3.12 NHL HOCKEY

You may experience some problems with NHL Hockey when using EMM386 as your

memory manager. If this happens you will need to use a third party memory

manager such as QEMM or 386MAX.

#6.0.0 ***** CHAPTER 6. - SOLUTIONS TO COMMONLY ENCOUNTERED PROBLEMS ******

PROBLEM:  When using Mega-Em, the music sounds choppy or there is a lot of

static and clicks.

SOLUTION: Many systems allow the ISA bus clock speed to be increased

beyond the standard speed of 8 MHz. The Gravis UltraSound is

designed to conform to the standard, and will not work reliably

on systems with high bus clock speeds. You should be able to

reduce the bus clock speed in one of your CMOS setup menus. The

CMOS setup is usually accessible by rebooting your computer.

You may have other software which works perfectly with a high

bus speed setting. There are two reasons why you may be able to

use them, yet not Mega-Em. Firstly a large amount of UltraSound

software is written in high level languages such as C. Mega-Em

is written in assembler which generally results in more

efficient (faster) code. Secondly Mega-Em is executed in

protected mode which results in I/O instructions being executed

faster than in virtual 8086 mode or real mode. Both of these

situations lead to the UltraSound being unable to 'keep up' with

Mega-Em.

PROBLEM:  Standard system information programs fail to show Mega-Em as

being present in memory. Examples of such programs include the

DOS MEM command, Nortons System Information, Quarterdeck

Manifest etc.

SOLUTION: Since Mega-Em installs in a non-standard way beyond the first

megabyte of memory, the standard DOS memory allocation system

will not know of it's presence. You should however see a drop

in free EMS memory after Mega-Em has been loaded, depending on

which memory manager you are using. The simplest way to see if

Mega-Em is installed in memory is to type EMUSET -? <ENTER>. If

Mega-Em is installed, you will see a help screen. If Mega-Em is

not installed you will see an error message stating this.

PROBLEM:  When I have Mega-Em loaded I get annoying pauses while running

certain software.

SOLUTION: This problem is most prominent with software which uses XMS/EMS

memory. It appears to caused by limitations of the 386/486

paging (virtual memory) system, and hence a side effect of

Mega-Em being resident and active in memory. There are several

work arounds you may try. Disable or remove any TSR programs

which use XMS/EMS such as disk caches. Limit your program's use

of XMS/EMS memory, or disk access.

Technical explanation: Memory managers use the 386/486 paging

mechanism to provide services such as EMS memory and upper

memory. Mega-Em loads high (above 8Mb) in the virtual memory

space to avoid conflicts with the memory manager. It appears

that the design of the paging mechanism is prone to the

occasional glitch, resulting in dramatic loss of speed. For

example, loading Mega-Em above the 32Mb mark in the virtual

memory space causes noticeable (on some systems intolerable)

slow-downs in the system speed. In theory this should not

happen. To back this up, the processors have special registers

soley for testing the paging mechanism, indicating the designers

were expecting problems. The only recourse would be to load

Mega-Em below the first megabyte of memory, but this would

however use an unpractical amount of precious DOS memory.

#7.0.0 ********* CHAPTER 7. - ANSWERS TO COMMONLY ASKED QUESTIONS *********

QUESTION: Could Mega-Em be modified so it does not require a 386 memory

manager, and hence work with software such as Comanche,

Ultima 7 etc. ?

ANSWER:   No. Mega-Em requires 386 protected mode for it's emulation to

operate. It uses the memory manager as a means of entering

protected mode, while still remaining compatible with a wide

variety of software. Mega-Em can not work with the above

mentioned software because they can not coexist with other

protected mode software. Furthermore, despite being able to run

with Mega-Em in memory, programs such as Strike Commander will

not work with Mega-Em because they take control of the 386

protected mode system, rendering Mega-Em inactive.

QUESTION: Well how come SBOS works with the above mentioned games?

ANSWER:   While SBOS is indeed a software emulator, it is hardware

assisted by specific features built into the UltraSound card.

The features will invoke SBOS when ever a program accesses

certain Sound Blaster port addresses. Mega-Em's emulation is

100% software, using special protection features of 386

protected mode.

QUESTION: Could features be added to Mega-Em to allow the UltraSound MIDI

port to be used as a Roland MPU-401 MIDI interface?

ANSWER:   Yes. This is already planned for future releases of Mega-Em.

QUESTION: Can I have the source code?

ANSWER:   No.

QUESTION: Why doesn't Mega-Em support OS/2?

ANSWER:   Mega-Em requires operating system privileges to function. OS/2,

like many multitasking operating systems will not allow DOS

programs to gain these. Most DOS memory managers will. Note that

it appears a suitable OS/2 driver would suffice. So anyone

contemplating writing OS/2 drivers for the UltraSound should

consider implementing similar features to Mega-Em.

#8.0.0 ************* CHAPTER 8. - MEGA-EM REVISION HISTORY ****************

#VERSION 2.03 12/22/93

* EMUSET -OFF no longer returns a "Failed to detect Sound Blaster base

address" error.

#VERSION 2.02 12/22/93

* You no longer need to use the -X2 switch with EMUSET for Mega-Em to

work with Street Fighter 2 (digital).

* Removed beta information from the header of this file.

#VERSION 2.01 12/13/93

* The master volume switch (-Vxx) now works.

* Corrected problems with Irq 2 and EMM386.

* EMUSET will now check to make sure Sound Blaster Irq's are working.

* Added a general purpose check for other resident UltraSound software.

* Added -FAST switch to EMUSET for problems with Sierra software on fast

machines.

* Added support for Netroom 3 (not compatible with Netroom's cloaking).

#VERSION 2.00B023 (Beta) 11/15/93

* Corrected lockup problems with the latest SBDIG.ADV drivers.

#VERSION 2.00B022 (Beta) 11/12/93 (No Public Release)

* Added master volume control (-Vxx) to EMUSET.

* Changed default emulation from MT-32 to Sound Canvas.

#VERSION 2.00B021 (Beta) 11/03/93 (No Public Release)

* Now automatically removes SBOS and UltraMID from memory.

* Corrected bug with MIDI port initialization.

* Corrected bug with DMA and Irq latch priming on UltraSound revisions 3.4

and later.

* Now handles ULTRASND.INI files with patch banking definitions.

#VERSION 2.00B020 (Beta) 10/22/93 (No Public Release)

* Removed demonstration period since Mega-Em is no longer shareware.

* Improved speed of FM register emulation routines.

* Added support for a few pieces of software which use non standard and

rare Sound Blaster DAC routines.

* Modified the music volume control so it now has a more natural response.

#VERSION 1.10B019 (Beta) 10/13/93 (No Public Release)

* Improved speed of Sound Blaster 'Direct DAC' routines by 50%.

* Added support for 2.6 and 2 bit compressed samples.

* Sound Blaster compressed samples now decompress in the background to

prevent annoying pauses.

* Added routines to remove most of the clicking in the Sound Blaster DMA

playback routines. Special enhancements remove clicks that occur when

using a real Sound Blaster card.

* Corrected bugs in the Sound Blaster DMA routines where Mega-Em

would not play the entire sample when very small DMA buffers were used.

* Default music volume is now lower to better suit the Sound Blaster DAC

emulation.

#VERSION 1.10B018 (Beta) 09/30/93 (No Public Release)

* Better compatibility and cleaner performance from the Sound Blaster DAC

emulation. Some techniques still need to be implemented to reduce

clicking.

* Added routines for playback of Sound Blaster 2:1 compressed samples.

#VERSION 1.10B017 (Beta) 09/25/93 (No Public Release)

* Implemented many of the Sound Blaster DAC features. Now works with

around 80% of Sound Blaster software. Still needs a lot of fine tuning.

#VERSION 1.10B016 (Beta) 09/09/93 (No Public Release)

* Modified initial state of the virtual Roland MIDI interface. Previously

the Roland auto-detect on some Lucas Arts adventures would fail.

* Added basic support for Sound Blaster DAC emulation. Works only with

'Direct DAC' software.

#VERSION 1.00  08/29/93

* Mega-Em will now sustain instruments properly. To counter for stuck

notes the /SS option has been added to EMUSET.

#VERSION 1.00B015 (Beta)  08/22/93

* Sysex handling routines have been rewritten for improved speed.

* Modified protected mode interrupt handlers for increased stability.

Should cure problems with emulation failing on some software.

* Added support for UltraSound 'backward' samples.

* At requests from users, Mega-Em will now display a message to say it is

registered.

* EMUSET will now handle ULTRASND.INI files which have been edited with a

text editor which adds an EOF character to the end of the file.

* Several minor touch ups to the UltraSound playback routines.

#VERSION 1.00B014 (Beta)  08/16/93

* Fixed a minor bug in the enveloping routines which caused roll-under

(i.e. the volume would suddenly go very loud) when certain instruments

were played very softly. This has also reduced clicking slightly.

* Added /M option to EMUSET to specify the music output volume.

* Added Pitch Wheel support. The implementation uses a fixed sensitivity

of two semitones. The sensitivity has been fixed because of speed

considerations.

* Mega-Em should now be stable when running under 386MAX.

* Fixed a bug in the uninstallation routines which caused problems with

386MAX (and possibly other memory managers).

* Increased demonstration time to 20 minutes.

#VERSION 1.00B013 (Beta)  08/10/93

* Fixed problems with the anti-debugger routines incorrectly identifying

386 systems as being in 'no privilege' virtual 8086 mode.

* Changed protected mode installation routines slightly. The /NGDT switch

has been removed and the /RGDT has been added. See chapter 4 for

details.

#VERSION 1.00B012 (Beta)  08/09/93

* Due to several 'cracks' appearing for Mega-Em's registration system,

Mega-Em is now more resistant against reverse engineering techniques.

Side effects include a short pause as Mega-Em loads, larger executable

size, slower performance for unregistered users and a low tolerance of

memory resident debuggers.

* Added /NOEMS switch to MEGAEM.EXE, forcing Mega-Em to load if EMS has

been disabled. Useful for users who disable the EMS page frame for

extra upper memory, however requires some special considerations. See

chapter 4 for details.

* The UltraSound playback routines are now stereo. The /MONO switch has

been added to disable this, as the occasional piece of music can cause

audible crackling during very fast stereo pans.

* Made extensive modifications to Mega-Em's protected mode installation

routines. Mega-Em should now be exhibit improved compatibility and

stability with a wider range of memory managers and computer systems.

For stubborn cases two new MEGAEM.EXE options, /FGDT and /NGDT, have

been added. See chapter 4 for details.

* Temporarily removed support for 386MAX until stability can be guaranteed

when using Mega-Em. Currently the system is prone to potentially

dangerous system crashes when Mega-Em is loaded with 386MAX.

* Added many enhancements to Mega-Em's protected mode interrupt handlers.

This has improved stability and now allows Irq 5 to be used for the

UltraSound.

* Added /F switch to EMUSET to allow the user to specify the sampling rate

at which Mega-Em stores patches in the UltraSound DRAM. 44 uses the full

sampling rate of the UltraSound patch. 22 strips the sampling rate in

half, resulting in approximately twice as many patches being loaded,

however also results in noisier playback.

* Added /MEM switch to MEGAEM.EXE to allow users to manually specify the

amount of memory on their UltraSound card.

* Improved enveloping routines for UltraSound non-looped percussion

patches.

#VERSION 1.00B011 (Beta)  07/26/93 (No Public Release)

* Changed the memory allocation routines slightly. Stops Mega-Em from

'thrashing' the 386 paging mechanism, which was causing noticeable speed

slowdowns. This has also removed some incompatibilities introduced with

version 1.00B008. The latter appears to be caused by a 386 protected

mode processor bug.

* Added support for several MT32 sysex messages, including channel note

off and reset.

* Due to several programs still leaving 'stuck notes', all UltraSound

sustained instruments will now slowly fade away.

#VERSION 1.00B010 (Beta)  07/25/93 (No Public Release)

* Fixed problems in routines which set sample start, loop start and loop

end. Previously Mega-Em would play from the wrong portion of the

UltraSound's memory on some samples.

* Improved the UltraSound enveloping routines. Patches such as

APPLAUSE.PAT now play correctly.

#VERSION 1.00B009 (Beta)  07/22/93

* Fixed problems in version 1.00B008 with emulation failing on many

systems (except the development system of course). This was related to

the bugs regarding the /CO1 and /CO2 options, which should also work

correctly now.

* Added the /L switch to EMUSET. This enables the UltraSound line in.

Useful if using Mega-Em in conjunction with a real Sound Blaster card.

* The UltraSound patch set is now loaded at full sampling rate

(approximately 44kHz) for 512k users. In previous versions the sampling

rate was halved (approximately 22kHz) resulting in slightly noisy

output. The sampling rate is still halved when using 256k.

#VERSION 1.00B008 (Beta)  07/20/93

* Temporarily removed the 10 minute demonstration period, until the

UltraSound playback routines sound better.

* Experimented with the playback of the UltraSound patches. Patches should

now play on key, however the envelope on many percussion patches is

worse than previous versions of Mega-Em.

#VERSION 1.00B007 (Beta)  07/16/93 (No Public Release)

* Added /CO1 and /CO2 options to EMUSET, allowing Mega-Em to be used in

conjunction with a real MPU-401 MIDI interface.

* Slowed down certain UltraSound playback routines. On some systems the

routines were too efficient (fast) for the UltraSound to keep up.

#VERSION 1.00B006 (Beta)  07/14/93

* Fixed problems on some machines with EMUSET crashing at the end of the

UltraSound MIDI patch load. This was caused by a 'POPAD bug' present in

many 386 and 486 processors.

* EMUSET will now default to C:\ULTRASND\MIDI or (ULTRADIR)\MIDI if there

is no PatchDir definition in ULTRAMID.INI.

* Several minor bugs fixed in the error handling routines.

#VERSION 1.00B005 (Beta)  07/13/93

* Fixed ULTRASND environment variable checking. Mega-Em will now load with

the UltraSound Record DMA set to something other than 1.

#VERSION 1.00B004 (Beta)  07/12/93

* First Public Release. Features Roland MT-32 and Sound Canvas emulation

using a Gravis UltraSound card as the output device.

#                                  THE END